| # Model | # 1717L |
| --- | --- |
| Color | Black or white |
| Active Display Area | 13.3" x 10.64" / 337.92mm x 270.34mm |
| Dimensions | 15.43" x 8.38" x 14.24" / 391.9mm x 215.4mm x 361.6mm See dimensional drawings for details |
| LCD Technology | Active matrix TFT LCD |
| Diagonal Size | 17” |
| On-Screen Display (OSD) | Accessible through switches along the bottom. Controls: Menu, Up, Down, Select Settings: Brightness, Contrast, Clock, Phase, H-position, V-position, Auto-Adjust, Aspect Ratio, Sharpness, Color Temperature, OSD Timeout, OSD Language, Recall Defaults Languages: English, French, Italian, German, Spanish, Simplified Chinese, Traditional Chinese, Japanese Lockouts: OSD, Power |
| Mounting Options | VESA 4-hole 75mm mounting interface on rear of unit 3-hole threaded mounting on bottom of stand See dimensional drawings for details |
| Aspect Ratio | 5:4 |
| Native Resolution | 1280 x 1024 |
| Preset Video Timings | 640x480 60Hz,75Hz 720x400 70Hz 800x600 56, 60Hz,75Hz 1024x768 60Hz,75Hz 1280x1024 60Hz,75Hz |
| Number of Colors | 16.7 million |
| Brightness (typical) | LCD Panel: 250 nits with IntelliTouch Zero-Bezel: 225 nits with IntelliTouch: 225 nits with AccuTouch: 200 nits with TouchPro PCAP: 225 nits |
| Response Time (Tr + Tf) (typical) | 5 msec |
| Viewing Angle (typical) | Horizontal: ±85° / 170° Vertical: ±80° /160° |
| Contrast Ratio (typical) | 1000:1 |
| Video | Analog VGA on Female DB-15 connector Input Video Horizontal Sync frequency range: 31.5 – 80KHz Input Video Vertical Sync (frame rate) frequency range: 56-75Hz |
| Power | Monitor input power connector - AC: IEC 60320 C6 Monitor input power signal specifications: 100-240VAC, 50/60Hz Monitor input power connector - DC: Coaxial power jack (2.0mm pin outer diameter, 6.4mm barrel inner diameter) Monitor input power signal specifications: 12VDC +/- 5% Note: Optional AC-DC power adapter sold for this monitor: Adapter input connector: IEC 60320 C14 Adapter input voltage range: 100-240VAC, 50/60Hz Power Consumption (Typical): ON (monitor only): 16W SLEEP (monitor only): 3W OFF (monitor only): 0.5W |
| Weight | Unpackaged: 11.9 lbs / 5.4 Kg Packaged: 18.3lbs / 8.3 Kg |
| Shipping Dimensions | 20.12" x 9.17" x 20.08" / 511mm x 233mm x 510mm |
| Temperature | Operating: 0° C to 40° C / 32° F to 104° F Storage: -20° C to 60° C / -4° F to 140° F |
| Humidity (non-condensing) | Operating: 20% - 80% Storage: 10% - 90% |
| Regulatory approvals and declarations | Argentina S-mark Australia C-Tick Canada cUL, IC China CCC Europe CE Japan VCCI Korea KCC, e-Standby Mexico CoC Taiwan BSMI Mark United States UL, FCC International CB RoHS, China RoHS, WEEE, IMERC |
| Warranty | 3 years |
| MTBF | 50,000 hours demonstrated |
| Box contents | Quick Install Guide Japan Class-B TM info sheet User Manual CD VGA Cable (DB-15 male to DB-15 male) Touch USB Cable (USB Type A plug to USB Type B plug) power cable, IEC 60320-C5 to NA/EU plug Stand Base assembly Filler Plate |
| Replacement DC Power Supply | The monitor may be powered with DC voltage. A optional 12V DC power brick, Elo P/N: E005277, is available at additional cost. |
| AC Power Cable | North American & EU power cables included. UK AC power cables included with P/Ns available for EMEA. China: E030476 Japan: E849156 (3-pin to 2-pin adapter: E351985 UK: E235317 Taiwan: E637694 |
| Cable Kit for Replacement Cables | Elo p/n - E378229, available at additional cost. |
| Optional PCAP Stylus Pen | E066148 |
| Touch/Video Cables | Detachable touch and video cables are included |
| Optional Peripherals | Magnetic Stripe Reader (Black MSR) – Elo P/N: E500356 Magnetic Stripe Reader (White MSR) – Elo P/N: E233227 |
| Stands, Mounts & Kits | 1717L VESA Mount Plate (White) - Elo p/n: E000421 1717L VESA Mount Plate (Black) - Elo p/n: E000405 1717 Replacement Stand (White) - Elo p/n: E061177 1717 Replacement Stand (Black) - Elo p/n: E904304 ELO-REAR-FACING-CD-BRKT-R - Elo p/n E332358 |

| # Part Number | # Part Description | # Touch Technology | # Surface Treatment | # Touch Interface | # Color |
| --- | --- | --- | --- | --- | --- |
| E649473 | ET1717L-7CWB-1-BL-ZB-G | AccuTouch Zero Bezel | Anti-glare | USB/Serial combo | Black |
| E679434 | ET1717L-7CWB-1-WH-ZB-G | AccuTouch Zero Bezel | Anti-glare | USB/Serial combo | White |
| E877820 | ET1717L-7CWB-1-BL-G | AccuTouch | Anti-glare | USB/Serial combo | Black |
| E017030 | ET1717L-8CWB-1-BL-ZB-G | IntelliTouch Zero-Bezel | Anti-glare | USB/Serial combo | Black |
| E179069 | ET1717L-8CWB-0-BL-ZB-G | IntelliTouch Zero-Bezel | Clear | USB/Serial combo | Black |
| E077464 | ET1717L-8CWB-1-BL-G | IntelliTouch | Anti-glare | USB/Serial combo | Black |
| E214112 | ET1717L-8CWB-1-WH-G | IntelliTouch | Anti-glare | USB/Serial combo | White |
| E111077 | ET1717L-7CDB-1-BL-ZB-G | AccuTouch Zero Bezel | Anti-glare | USB/Serial combo | Black |
| E108749 | ET1717L-8CDB-0-BL-ZB-G | IntelliTouch Zero-Bezel | Clear | USB/Serial combo | Black |
| E824217 | ET1717L-2UWB-1-BL-ZB-G | TouchPro PCAP | Anti-glare | USB | Black |

| Document Number | Description |
| --- | --- |
| MS601505 | Dimensional drawings |
| MD600055 | Regulatory certificates |
| SW601708 | User Guide – English |
| SW601709 | User Guide – French |
| SW601710 | User Guide – Italian |
| SW601711 | User Guide – German |
| SW601712 | User Guide – Spanish |
| SW601713 | User Guide – Japanese |
| SW601714 | User Guide – Simplified Chinese |
| SW601715 | User Guide – Traditional Chinese |
| SW601716 | User Guide – Korean |

## MTBF (Mean Time Between Failures) An MTBF measurement is based on a statistical sample and is not intended to predict any one specific unit's reliability; thus MTBF is not, and should not be construed as, a warranty measurement.