| # Model | # 1002L |
| --- | --- |
| Color | # Black |
| Diagonal Size | 10.1” / 256.54mm |
| Active Display Area | 8.54” x 5.34” / 216.96mm x 135.6mm |
| LCD Technology | Active matrix TFT LCD |
| Number of Touches | 10 Touches |
| Touch Technology | TouchPro PCAP |
| Aspect Ratio | 16:10 |
| Native Resolution | 1280 x 800 |
| Preset Video Timings | 640x480 60Hz 800x600 60Hz 1024x768 60Hz 1280x800 60Hz 1920x1080 60Hz 1920x1200 60Hz |
| Number of Colors | 262K |
| Brightness (typical) | Panel: 350 nits With PCAP + DC power: 315 nits With PCAP + USB-C 5V power: 200 nits |
| Response Time (Tr + Tf) (typical) | 29 msec |
| Viewing Angle (typical) | Horizontal: ±85° / 170° total Vertical: ±85° / 170° total |
| Contrast Ratio (typical) | 800:1 |
| On-Screen Display (OSD) | Accessible through switches along the bottom. Controls: Menu, Power, Up, Down, Select Settings: Brightness, Contrast, Clock, Phase, H-position, V-position, Auto-Adjust, Aspect Ratio, Sharpness, Color Temperature, OSD Timeout, OSD Language, Volume, Mute, Recall Defaults, Audio Select, Video Priority, Information, Touch on Sleep Mode, Peripheral Port Languages: English, French, Italian, German, Spanish, Simplified Chinese, Traditional Chinese, Japanese Lockouts: OSD, Power |
| Mounting Options | VESA 4-hole 75mm mounting interface on rear of unit See dimensional drawings for details |
| I/O Ports | 1 x USB Type-C 1 x Audio jack (Line-in) 1 x VGA (Micro-HDMI female connecter) 1 x HDMI 1 x 12V D/C input 3x Micro-USB ports (Top, Left, Right) to support Elo Edge Connect peripherals |
| Video | Analog VGA HDMI USB Type-C *Suggest using Elo USB Type-C to Type-C (Elo P/N: E710364) cable to prevent any compatibility issues Input Video Horizontal Sync frequency range: 31.5 – 80 KHz Input Video Vertical Sync (frame rate) frequency range: 60Hz |
| Power | Option 1: USB Type-C Input range: 5V 3A/9V 3A/15V 1.8A/20V 1.35A Option 2: External DC Adapter 12V/36W in box Monitor input power connector: Coaxial power jack (2.0mm (+0.0 -0.1mm) pin outer diameter, 6.4mm (+/- 0.3mm) barrel inner diameter, 8.8mm (+/- 0.3mm) Barrel depth) Monitor input voltage range: 12VDC +/- 5% Adapter output power connector: DC barrel plug (2.1mm (+/- 0.1mm) pin inner diameter, 5.5mm (+/- 0.05mm) barrel outer diameter, 9.5mm (+/- 0.3mm) Barrel length) Adapter input voltage range: 100-240 VAC Input frequency: 50-60 Hz Power Consumption (Typical): ON (monitor + adapter): 10W SLEEP (monitor + adapter): 1.5W OFF (monitor + adapter): 0.2W |
| Speakers | 2 x 2W internal speakers |
| Weight | Unpackaged Without Stand: 1.95lbs. /0.8842 kg Packaged Without Stand: 4.02 lbs. / 1.8266 kg |
| Dimensions (H x W x D) | Without Stand: 9.86'' x 6.51' x 1.12'' / 250.42mm x 165.31mm x 28.45mm Shipping： Without Stand: 12.4'' x 9.57' x 5.67'' / 315 mm x 243mm x 144mm |
| Temperature | Operating: 0° C to 40° C / 32° F to 104° F Storage: -20° C to 60° C / -4° F to 140° F |
| Humidity (non-condensing) | Operating: 20% - 80% Storage: 10% - 95% (38.7℃ max wet bulb temperature) |
| Regulatory approvals and declarations | Canada cUL, IC Europe CE Japan VCCI United States UL, FCC China CCC Korea, KCC, e-standby Australia RCM Mexico NOM Taiwan BSMI International CB RoHS, China RoHS, WEEE, REACH India BIS |
| Ingress Protection | IPX1 for front side |
| Compliance |  |
| Warranty | 3 years |
| Additional Service Offerings |  |
| Extended Warranty | 4 years warranty coverage - Elo P/N: E898247 5 years warranty coverage - Elo P/N: E898449 3 years warranty coverage + AUR - Elo P/N: E898648 4 years warranty coverage + AUR - Elo P/N: E894921 5 years warranty coverage + AUR - Elo P/N: E895125 |
| Other Features |  |
| Sealing | IPX1 for front side |
| What's in the Box | Touchmonitor HDMI cable USB Cable (USB Type-A to Type-C) Quick Install Guide EEI Label Power brick NA Power cable EU Power cable 2x cable clamps 2x tie wraps 3x screws (M3x8mm) for cable clamps 2x screws (M3x10mm) for cable doors |
| MTBF | 50,000 hours demonstrated |

| # Part Number | # Part Description | # Touch Technology | # Surface Treatment | # Touch Interface | # Color |
| --- | --- | --- | --- | --- | --- |
| E155834 | ET1002L-2UWC-1-G | PCAP | Antiglare | USB | Black |
| E324341 | ET1002L-0NWC-1-ZB-GY-G | No Touch | Antiglare | USB | Black |

| Options/Peripheral/Upgrades | Part # | Options/Peripheral/Upgrades | Part # |
| --- | --- | --- | --- |
| Magnetic Stripe Reader | E001002 | Rear Facing Customer Display Bracket | E332358 |
| Barcode Reader | E093433 | Shelf Mount | E043382 |
| NFC reader | E001004 | Pole Mount Bracket | E045151 |
| VFD Customer Display | E001003 | Table Top Stand | E160104 |
| Pole mount – 6 inch | E047458 | ELO-KIT-CABLE-USB-C-to-USB-C | E710364 |
| Pole mount – 12 inch | E047663 | ELO-KIT-CABLE-VGA-DB15-to-MICRO | E710549 |
| Pole mount – 18 inch | E047864 | ELO-KIT-CABLE-DVI-D-To-HDMI | E538507 |
| Floor Stand – 5 foot | E048069 | ELO-KIT-ANDROID-MEDIA-PLAYER | E611864 |
| Wallaby™ POS Stand (Black) | E949536 | Wallaby™ POS Stand (White) | E353758 |
| 2D Barcode Scanner | E926356 | Wall Mount Kit (75mm or 100mm VESA) | E143088 |
| Elo Backpack® 3.0 | E901355 | EMV Cradle for Ingenico ICMP (I-Series & 02-Series Monitors) | E200465 |
| EMV Cradle for Verifone E355 (I-Series & 02-Series Monitors) | E201088 | EMV Cradle for MagTek eDynamo | E375343 |
| EMV Cradle for Ingenico RP457c (with Audio Jack, BT and USB) | E586981 | EMV Cradle for Ingenico RP457c (with BT and USB) | E710930 |

| Document Number | Description |
| --- | --- |
| SW601743(E893866) | QIG |
| MS602564 | Dimensional drawings |
| MD600067 | Regulatory certificates |
| UM600243 | User Guide – English |
| UM600244 | User Guide – French |
| UM600245 | User Guide – Italian |
| UM600246 | User Guide – German |
| UM600247 | User Guide – Spanish |
| UM600248 | User Guide – Japanese |
| UM600249 | User Guide – Simplified Chinese |
| UM600250 | User Guide – Traditional Chinese |
| UM600251 | User Guide – Russian |
| UM600252 | User Guide – Korean |

## MTBF (Mean Time Between Failures) An MTBF measurement is based on a statistical sample and is not intended to predict any one specific unit's reliability; thus MTBF is not, and should not be construed as, a warranty measurement.