| # Model | # 2002L |
| --- | --- |
| Color | Black |
| Active Touch Area | 436.9 x 240.7 mm 17.20 x 9.48 in |
| LCD Technology | Active matrix TFT LCD |
| Diagonal Size | 19.5” |
| Number of Touches | 10 Touches |
| On-Screen Display (OSD) | Accessible through switches along the bottom. Controls: Menu, Power, Up, Down, Select Settings: Brightness, Contrast, Clock, Phase, H-position, V-position, Auto-Adjust, Aspect Ratio, Sharpness, Color Temperature, OSD Timeout, OSD Language, Volume, Mute, Recall Defaults, Audio Select Languages: English, French, Italian, German, Spanish, Simplified Chinese, Traditional Chinese, Japanese Lockouts: OSD, Power |
| Mounting Options | VESA 4-hole 75mm and 100mm mounting interface on rear of unit See dimensional drawings for details |
| Aspect Ratio | 16:9 |
| Native Resolution | 1920 x 1080 |
| Preset Video Timings | 640 x 480 60Hz 640 x 400 70Hz 720 x 576 50Hz 720 x 400 70Hz 800 x 600 56, 60Hz 1024 x 768 60Hz 1280 x 720 50, 60Hz 1280 x 960 60Hz 1280 x 1024 60Hz 1366 x 768 60Hz, 60Hz RB 1440 x 900 60Hz, 60Hz RB 1680 x 1050 60Hz, 60Hz RB 1920 x 1080 50, 60Hz |
| Number of Colors | 16.7M |
| Brightness (typical) | Panel: 250 nits With PCAP: 225 nits |
| Response Time (Tr + Tf) (typical) | 20 ms |
| Viewing Angle (typical) | Horizontal: 178° Vertical: 178° |
| Contrast Ratio (typical) | 3000:1 |
| Video | Mini-VGA (Mini-VGA to VGA DE-15 (DB-15) Male connecter cable included) HDMI (HDMI cable included) Input Video Horizontal Sync frequency range: 31.5 – 80KHz Input Video Vertical Sync (frame rate) frequency range: 50 - 60Hz |
| Power | Monitor input power connector: Coaxial power jack (2.5 mm pin outer diameter, 6.4mm barrel inner diameter) Monitor input power signal specifications: 12VDC +/- 5% Adapter input voltage range: 100-240VAC, 50/60Hz Power Consumption (Typical): ON (monitor + AC/DC adapter): 18.4 W SLEEP (monitor + AC/DC adapter): 0.33 W OFF (monitor + AC/DC adapter): 0.24 W |
| Speakers | 2 x 2W internal speakers |
| Dimensions (H x W x D) | With Stand: 13.87” x 19.22” x 7.61” / 352.3 x 488.31 x 193.4 mm Without Stand: 11.57” x 19.22” x 1.52” / 293.9 x 488.31 x 38.7 mm Shipping: With Stand: 16.50” x 22.52” x 9.92” / 419 x 572 x 252 mm Without Stand: 18.35” x 24.49” x 7.17” / 466 x 622 x 182 mm |
| Weight | With Stand: 15lbs / 6.8kg Without Stand: 7.25 lbs / 3.29 kg Shipping: With Stand: 19.84lbs / 9kg Without Stand: 11.42 lbs / 5.18 kg |
| Temperature | Operating: 0° C to 40° C / 32°F to 104°F Storage: -20° C to 60° C / -4°F to 140°F |
| Humidity (non-condensing) | Operating: 20% - 80% Storage: 10% - 95% |
| Regulatory approvals and declarations | Canada cUL, IC Europe CE Japan VCCI United States UL, FCC China CCC Korea, KCC, e-standby Australia RCM India BIS |
| Warranty | 3 years 4 year warranty coverage - Elo P/N: E898247. 5 year warranty coverage - Elo P/N: E898449. 3 year warranty coverage + AUR - Elo P/N: E898648. 4 year warranty coverage + AUR - Elo P/N: E894921. 5 year warranty coverage + AUR - Elo P/N: E895125. |
| MTBF | 50,000 hours demonstrated |
| What’s in the Box | 1 x Touchmonitor Mini-VGA to VGA cable (mini-VGA to DE-15 (DB-15) male) USB cable (USB Type A plug to USB Type B plug) HDMI cable Audio cable User manual CD QIG Japan class B TM info sheet Power brick with US, EU, UK, KR, AU and CN clips 4 x Screws (Only for E125897) |

| # Part Number | # Part Description | # Touch Technology | # Surface Treatment | # Touch Interface | # Color |
| --- | --- | --- | --- | --- | --- |
| E396119 | ET2002L-2UWA-1 -G | PCAP | Antiglare | USB | Black |
| E125897 | ET2002L-2UWA-1-NS-G | PCAP | Antiglare | USB | Black |

| Document Number | Description |
| --- | --- |
| MS601774 | Dimensional drawings |
| MD600068 | Regulatory certificates |
| UM600012 | User Guide – English |
| UM600013 | User Guide – French |
| UM600014 | User Guide – Italian |
| UM600015 | User Guide – German |
| UM600016 | User Guide – Spanish |
| UM600017 | User Guide – Japanese |
| UM600018 | User Guide – Simplified Chinese |
| UM600019 | User Guide – Traditional Chinese |
| UM600151 | User Guide – Korean |

## MTBF (Mean Time Between Failures) An MTBF measurement is based on a statistical sample and is not intended to predict any one specific unit's reliability; thus MTBF is not, and should not be construed as, a warranty measurement.

| Options/Peripheral/Upgrades | Part # |
| --- | --- |
|  | (RoHS Compliant) |
| 1.8m Cable Kit | E210789 |
| 3m Cable Kit | E210606 |
| Magnetic Stripe Reader | E001002 |
| Barcode Reader | E093433 |
| NFC reader | E001004 |
| VFD Customer Display | E001003 |
| Pole mount – 6 inch | E047458 |
| Pole mount – 12 inch | E047663 |
| Pole mount – 18 inch | E047864 |
| Floor Stand – 5 foot | E048069 |
| Power Brick Kit - 1.8m Power Cable | E211174 |
| Power Brick Kit - 3m Power Cable | E210973 |
| Wall mount | E143088 |
| Shelf Mount | E043382 |
| Pole Mount Bracket | E045151 |
| Table top dual hinge stand | E044162 |

| - Magnetic Stripe Reader (E001002) |  |
| --- | --- |
| - Swipe Speed | - 4~60 ips (10.1~152.4 cm/sec) |
| - Swipe direction | - Bi-direction |
| - Card Width | - 0.025 ~0.035 inches |
| - Number of Tracks | - 3 tracks |
| - Encryption | - Non-Encrypting Level 2 |
| - Power Supply | - 5.0VDC +/- 0.25V |
| - Power Consumption | - < 2.0 W |
| - Interface | - USB |
| - Supported Emulation Modes | - HID and HID Keyboard |
| - Supported Magnetic Stripe Formats | - ISO 7811 - ISO 7810 - AAMVA - CA DMV - ANSI |
| - Supported Operating Systems | - Ubuntu Linux LTS - Windows POSReady 7 32-bit - Windows Embedded Pro 7 32-bit - Windows Embedded Pro 7 64-bit - Windows Industry Embedded Pro 8.1 32-bit - Windows Industry Embedded Pro 8.1 64-bit - Android 4.4.2 |

| - NFC reader (E001004) |  |
| --- | --- |
| - Power Supply | - 5 VDC |
| - Interface | - USB |
| - Interface Modes | - HID Keyboard and USB Virtual COM |
| - Communication Range | - 40mm |
| - Operating modes | - Read/Write - Peer-to-Peer - Card emulation |
| - Frequency Range | - 13.56 MHz |
| - Supported Operating Systems | - Windows POSReady 7 32-bit - Windows Embedded Pro 7 32-bit - Windows Embedded Pro 7 64-bit - Windows Industry Embedded Pro 8.1 32-bit - Windows Industry Embedded Pro 8.1 64-bit - Android 4.4.2 |

| - VFD Customer Display (E001003) |  |
| --- | --- |
| - Display Type | - Vacuum fluorescent display (VFD) |
| - Display Color | - Green |
| - Brightness | - 500-1000 cd/m2 |
| - Characters Available | - 95 alphanumeric & 32 international characters |
| - Dot Size (X x Y) | - 0.86 x 1.2mm |
| - Font Size (W x H) | - 5.5 x 10.5 |
| - Character Number | - 20 characters by 2 lines |
| - Interface | - USB |
| - Viewing Angle | - Max. 90° |
| - Panel Dimension | - 220(W) x 50(D) x87(H) mm |
| - Power Supply | - 5.0 VDC |
| - Power Consumption | - 2.0 W |
| - Supported Command Types | - ICD2002 - EPSON POS D101 - UTC Standard - UTC enhance - AEDEX - ADM788 - DSP800 - CD5220 |
| - International Character Set - ( Code 20H-7FH) | - Code 20H-7FH) - Note - U.S.A (PC-437 USA） - FRANCE (PC-850) - GERMARY (PC-850) - U.K. (PC-850) - DENMARK I (PC-850) - DENMARK II (PC-850) - SWEDEN (PC-850) - ITALY (PC-850) - SPAIN (PC-850) - JAPAN (Katakana) - NORWAY (PC-850) - SLAVONIC - RUSSIAN |
| - MTBF | - 30,000 hours |
| - Supported Operating Systems | - Windows POSReady 7 32-bit - Windows Embedded Pro 7 32-bit - Windows Embedded Pro 7 64-bit - Windows Industry Embedded Pro 8.1 32-bit - Windows Industry Embedded Pro 8.1 64-bit - Android 4.4.2 |

| - Barcode Reader (E093433) |  |
| --- | --- |
| - Interface | - HID USB |
| - Ideal scan distance | - 110 mm |
| - Reader type | - 1D barcodes |
| - Supported 1D Symbologies | - UPC-A, UPC-E, EAN13, ENA8, CODE39, CODE93, CODABAR, CODE128, MSI/Plessey, CODE 32, CODE 11, TELEPEN, GS1 DataBar Ominidirectional, GS1 DataBar Limited, GS1 DataBar Expanded, Interleaved 2 of 5, Industrial 2 of 5, Matrix 2 of 5 |