| TITLE: PRODUCT SPECIFICATIONS, i-series 3.0 with Google Play Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| NUMBER: PS600376 |  |  |  |  |
| REVISION: B |  |  |  |  |
| OWNER: Engineering |  |  |  |  |
| Change History |  |  |  |  |
| Rev | Author | Date | Description |  |
| A | Asela E | 03/21/2019 | Initial release per CO-19-0532 |  |
| B | Asela E | 04/15/2019 | Update per: Add camera Add value information |  |
| B2 | Josh | 7/31/2019 | Update Warranty PNs |  |
| B3 | Josh | 8/8/2019 | Add country list |  |
| B4 | Josh | 8/12/2019 | Add “with Google Play Services” in OS |  |

| Model | i-series 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size (In) (mm) | 10.1 256.54 | 15.6 396.24 |  |  | 21.5 546.10 |
| Processor | Qualcomm Snapdragon APQ8053 2.0GHz Octa-Core Processor |  |  |  |  |
| Memory (RAM) | 3GB for Standard Versions DDR3L 2GB for Value Versions DDR3L |  |  |  |  |
| Storage (emmc 5.1) 2GB equals one billion bytes when referring to storage capacity. Accessible capacity may be less. | 32GB SSD for Standard versions 16GB SSD for Value versions |  |  |  |  |
| IO Ports | Standard Micro HDMI output Micro SD slot Ethernet 1x LAN (Gigabit) GPIO 1x USB 3.0 support (900mA) Value Ethernet 1x LAN (Gigabit) 1x USB 3.0 support (900mA) Micro SD slot GPIO |  |  |  |  |
| Number of Touches | Up to 10 |  |  |  |  |
| Wireless | 802.11b/g/n/ac + Bluetooth 4.1 |  |  |  |  |
| Peripherals Mounting (all micro-USB port are 2.0 ports and support 500mA) | 4x micro-USB side peripheral ports available for the standard version Not available on value SKUs |  |  |  |  |
| Webcam Not available on the Value SKU | Auto exposure Auto Gain Control Sharpness/Brightness/Contrast control Max resolution: 1920 x 1080 30fps |  |  |  |  |
| Speakers | 2x2W Stereo internal speakers |  |  |  |  |
| Audio | Standard version has a built-in microphone External microphone and headphone support are available via an optional the USB adaptor (Vantech NBA-120U). |  |  |  |  |
| Operating System Options | Android 8.1TM OreoTM with Google Play Services; EloView content deployment and management |  |  |  |  |
| Aspect Ratio | 16:10 | 16:9 |  |  | 16:9 |
| Panel Active Display Area (mm) (in) | 217 x 136 8.54 x 5.34 | 344 x 194 13.55 x 7.62 |  |  | 476 x 268 18.74 x 10.54 |
| Native (optimal) Resolution | 1280 x 800 @ 60Hz | 1920 x 1080 @ 60Hz |  |  |  |
| Brightness (typical) LCD Panel With TouchScreen | 400 344 | Std: Val: 300 220 258 189 |  |  | 250 215 |
| Number of Colors | 8-bit (16.7M) | 6-bit (262K) |  |  | 6-bit (Hi-FRC; 16.7M) |
| Response Time (Tr + Tf) (typical) | 25 ms | Std: Val: 30 ms 12 ms |  |  | 14 ms |
| Viewing Angle (typical) Horizontal Vertical | ± 85° ± 85° | Std: Val: ± 85° ± 45° ± 85° 20° and 40° |  |  | ± 89° ± 89° |
| Contrast Ratio (typical) | 800 | Std: Val: 800 500 |  |  | 1000 |
| Available Touch Technologies | Touch Pro Film (Pro-F) |  |  |  |  |
| Power Input | AC input voltage: 100-240 VAC Input frequency: 50-60 Hz |  |  |  |  |
| Power Consumption (watts) Typical On Off Sleep (Idle Mode) | 10.2 0.4 5.0 | 13 0.4 4.5 |  |  | 16 0.5 4.0 |
| Power Over Ethernet (PoE) | Supported via Elo PoE Device PN: E615169 Reference the I-Series 3.0 user guide for details. |  |  |  |  |
| Maximum System Dimensions (W x H x D) mm (W x H x D) in | 255 x 181 x 25 10.06 x 7.13 x 0.99 | 387 x 248 x 28 15.23 x 9.77 x 1.11 |  |  | 524 x 324 x 35 20.63 x 12.75 x 1.39 |
| Shipping Box Dimensions (W x H x D) mm (W x H x D) in | 330 x 250 x 140 12.99 x 9.84 x 5.51 | 150 x 300 x 470 5.90 x 11.81 x 18.50 |  |  | 180 x 400 x 600 7.08 x 15.75 x 23.62 |
| Weight Kgs Lbs | 0.88 1.94 | 1.59 3.50 |  |  | 4.30 9.47 |
| Shipping Weight Kgs Lbs | 2.07 4.56 | 3.27 7.20 |  |  | 7.30 16.10 |
| Temperature | Operating: 0°C to 40°C (32°F to 104°F) Storage: -20°C to 50°C (-4°F to 122°F) |  |  |  |  |
| Humidity (non-condensing) | Operating: 20% to 80% Storage: 5% to 95% |  |  |  |  |
| Regulatory approvals and declarations (Google CTS versions) | UL/cUL (US) FCC (US) IC (Canada) CE (Europe) CB (International) |  |  |  |  |
| Mounting Options | VESA 75mm x 75mm |  | VESA 75mm x 75mm and VESA 100mm x 100mm | VESA 100mm x 100mm |  |
| Enclosure Color | Black or White colors available |  |  |  |  |
| Real-Time Clock | Lithium coin cell battery for real time clock |  |  |  |  |
| Sealing | Touchscreen sealed to LCD |  |  |  |  |
| Power Cables (included in accessory box) | North American European |  |  |  |  |
| What’s in the box | i-series 3.0 device North American Power Cable European Power Cable 1x Power Brick (E153026) Quick Install Guide Screws for cable cover door (located in the power/Ethernet bay) |  |  |  |  |
| Warranty | 2 years |  |  |  |  |
| Extended Warranty Options | 3 year warranty coverage - Elo P/N: E819394 4 year warranty coverage - Elo P/N: E819598 5 year warranty coverage - Elo P/N: E819795 2 year warranty coverage + AUR - Elo P/N: E819985 3 year warranty coverage + AUR - Elo P/N: E820186 4 year warranty coverage + AUR - Elo P/N: E820388 5 year warranty coverage + AUR - Elo P/N: E820582 |  |  |  |  |
| Supported Country List | - Canada - USA - Australia - Japan - South Korea - New Zealand - Austria - Belgium - Czech Republic - Denmark - France - Germany - United Kingdom - Hungary - Italy - Netherlands - Poland - Portugal - Romania - Spain - Sweden - Switzerland - Turkey |  |  |  |  |

Part Numbers:

| Family | Elo P/N | Part Description | Color | Memory | Storage | Availability |
| --- | --- | --- | --- | --- | --- | --- |
| With GoogleTM Play Services | E461790 | ESY10I1-2UWB-0-AO-GY-G | Black | 3GB | 32GB | WW |
|  | E461993 | ESY10I1-2UWC-0-AO-GY-G |  | 2GB | 16GB |  |
|  | E462193 | ESY15I1-2UWB-0-AO-GY-G |  | 3GB | 32GB |  |
|  | E462384 | ESY15I1-2UWC-0-AO-GY-G |  | 2GB | 16GB |  |
|  | E462589 | ESY22I1-2UWB-0-AO-GY-G |  | 3GB | 32GB |  |

Optional Accessories

| Elo P/N | Part Description | Contents (works with all sizes unless specified) |
| --- | --- | --- |
| E001002 | KIT,MSR,USB, ESY X Series/AAiO/02 Series | MSR kit |
| E001004 | KIT,NFC,USB, ESY X Series/AAiO/02 Series | NFC kit |
| E093433 | KIT,BCR,USB, AAiO/02 Series | BCR kit (1D) |
| E926356 | ELO-KIT, 2DBCR, USB, N3680 | BCR kit (2D) |
| E211544 | KIT, GPIO Cables for AAiO | Black GPIO Cable |
| E048069 | ELO-KIT-FLOOR STAND 5 FOOT-R | Pole Mount Kit |
| E160104 | ELO-STAND-10.1IN-GY-R | 10.1 Stand Kit |
| E044162 | ELO-STAND-15.6IN-GY-R | 15.6 Stand Kit |
| E044356 | ELO-STAND-21.5IN-GY-R | 21.5 Stand Kit |
| E043382 | ELO-SHELFMNT-R | Shelf Mount Kit |
| E412240 | KIT,EXPANSION MODULE,ESY I-Series | Expansion Module Kit for 15.6 and 21.5 only |
| E923781 | KIT, EXP MODULE LONG CABLE, I-SERIES | Expansion Module Kit for 15.6 and 21.5 only |
| E924077 | ELO-STAND-FLIP DESIGN-10I/15I/1502/1002 | Flip Stand Kit for 10.1 and 15.6 only |
| E200465 | KIT-EMV-INGENICO ICMP CRADLE-I/M | EMV Cradle for 15.6 and 21.5 10.1 (landscape only) |
| E201088 | KIT-EMV-VERIFONE E355 CRADLE-I/M | EMV Cradle for 15.6 and 21.5 10.1 (landscape only) |
| E200788 | KIT-EMV-INGENICO ICMP CRADLE-X/IDS02 | EMV Cradle for 10.1 (portrait only) |
| E201363 | KIT-EMV-VERIFONE E355 CRADLE-X/IDS02 | EMV Cradle for 10.1 (portrait only) |
| E353950 | ELO-mPOS PRINTER STAND-I-SERIES-BK | Printer stand (black) for 10.1 and 15.6 |
| E353758 | ELO-mPOS PRINTER STAND-I-SERIES-WH | Printer stand (white) for 10.1 and 15.6 |
| E047458 | ELO-KIT-BOLT DOWN 6 INCH-R | 6in bolt down kit |
| E047663 | ELO-KIT-BOLT DOWN 12 INCH-R | 12in bolt down kit |
| E047864 | ELO‐KIT‐BOLT DOWN 18 INCH‐R | 18in bolt down kit |
| E802400 | KIT, WALLMOUNT, ESY I-Series 2.0 | Low profile wallmount kit (not compatible when the PoE is installed) |
| E615169 | ELO-KIT-POE-ADAPTER | POE module kit |
| E143088 | KIT, WALL MOUNT, ESYxx EX-Series | Will work with PoE |
| E514304 | KIT, REPLACEMENT CABLE, I-SERIES 2.0 | 1x NA power cable 1x EU power cable 1x power brick for i-series 2.0 |
| E001001 | KIT,FPR,USB,ESY X-Series | Fingerprint reader |

Documentation List

| Document Number | Description |
| --- | --- |
| MS601903 | DIMENSIONAL DRAWING, 10in AAiO_2 |
| MS601904 | DIMENSIONAL DRAWING, 15in AAiO_2 |
| MS601905 | DIMENSIONAL DRAWING, 22in AAiO_2 |
| QIG600031 | QIG, i-series 2.0 |
| UM600273 | USER GUIDE, Android I-Series 3.0 with CTS |
| UM600285 | USER GUIDE, Android I-Series 3.0 with CTS, FRENCH |
| UM600286 | USER GUIDE, Android I-Series 3.0 with CTS, GERMAN |
| UM600287 | USER GUIDE, Android I-Series 3.0 with CTS, ITALIAN |
| UM600288 | USER GUIDE, Android I-Series 3.0 with CTS, SPANISH |

## MTBF (Mean Time Between Failures) An MTBF measurement is based on a statistical sample and is not intended to predict any one specific unit's reliability; thus MTBF is not, and should not be construed as, a warranty measurement.

Copyright © 2019 Elo Touch Solutions, Inc. All Rights Reserved.

No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, including, but not limited to, electronic, magnetic, optical, chemical, manual, or otherwise without prior written permission of Elo Touch Solutions, Inc.

Disclaimer

The information in this document is subject to change without notice. Elo Touch Solutions, Inc. and its affiliates (collectively "Elo") makes no representations or warranties with respect to the contents herein, and specifically disclaims any implied warranties of merchantability or fitness for a particular purpose. Elo reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of Elo to notify any person of such revisions or changes.

Trademark Acknowledgments

AccuTouch, CarrollTouch, Elo (logo), Elo Touch Solutions, Elo TouchSystems, IntelliTouch, iTouch are trademarks of Elo and its Affiliates. Windows is a trademark of Microsoft Corporation.