| TITLE: PRODUCT SPECIFICATIONS, WINDOWS I-SERIES 2.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NUMBER: PS600399 |  |  |  |  |  |  |  |  |
| REVISION: A |  |  |  |  |  |  |  |  |
| OWNER: Engineering |  |  |  |  |  |  |  |  |
| Change History |  |  |  |  |  |  |  |  |
| Rev | Author | Date | Description |  |  |  |  |  |
| 0 | Josh Zhang | May 1, 2019 | - Initial draft |  |  |  |  |  |
| 1 | Chunying Huang | July 26, 2019 | - Fill out missing content from v0 product spec |  |  |  |  |  |
| A | Chunying Huang | Aug 12, 2019 | Initial release per CO-19- |  |  |  |  |  |

| Model | ESYxxI2 Model | ESYxxI3 Model | ESYxxI5 Model |
| --- | --- | --- | --- |
| Enclosure Color | Black |  |  |
| Active Display Area (W x H) | 15.6” models: 13.55" x 7.62" (344.2 mm x 193.5 mm) 21.5” models: 18.74” x 10.54” (476.0 mm x 267.8 mm) |  |  |
| Maximum Touchcomputer Dimensions (W x H x D) | 15.6” models: 15.13” x 9.7” x 1.72” (384.2 mm x 246.4 mm x 43.8 mm) 21.5” models: 20.7” x 12.57” x 2.07” (525.9 mm X 319.2 mm X 52.6 mm) |  |  |
| LCD Technology | Active matrix TFT LED LCD |  |  |
| Diagonal Size | 15.6” models: 15.55" (395 mm) diagonal 21.5” models: 21.53” (547 mm) diagonal |  |  |
| Processor | Intel® Celeron® J4105 Processor (4M Cache, up to 2.5 GHz) | Intel® Core™ i3-8100T Processor (6M Smart Cache, 3.10 GHz) | Intel® Core™ i5-8500T Processor (9M Smart Cache, up to 3.50 GHz by Intel® Turbo Boost Technology 2.0) Intel® vPro Support |
| Chipset | N/A | Intel H310 Express | Intel Q370 Express |
| Video | Intel UHD Graphics 600 | Intel UHD Graphics 630 |  |
| BIOS | AMI |  |  |
| Memory (RAM) | 4GB 2666MHz DDR4 SO-DIMM on 1 of 2 slots (Expandable to 8GB maximum1 on 2 slots)3 | 8GB 2666MHz DDR4 SO-DIMM on 1 of 2 slots (Expandable to 32GB maximum2 on 2 slots)3 |  |
|  | 1Maximum memory using 2 slots. Expandable up to 8GB maximum, SO-DIMM DDR4-2666. 3Check brand of the memory included in your unit before purchasing the memory kit. | 2Maximum memory using 2 slots. Expandable up to 32GB maximum, SO-DIMM DDR4-2666. 3Check brand of the memory included in your unit before purchasing the memory kit. |  |
| Storage | 128GB M.2 SATA SSD4 72 TBW 4GB equals one billion bytes when referring to storage capacity. Accessible capacity may be less. |  |  |
| Mounting Options | 15.6” models: 75mm VESA mount 21.5” models: 75mm or 100mm VESA mount |  |  |
| Ethernet Port | 1 x LAN RJ45 (Gigabit) |  |  |
| Wireless LAN | 802.11 a/b/g/n/ac 2.4GHz and 5GHz |  |  |
| Bluetooth | BT 5.0 |  |  |
| USB Ports | 1 x USB 3.0 4 x Micro USB 2.0 on edges for optional Elo peripherals. |  |  |
| RAID | N/A |  |  |
| TPM | TPM 2.0 |  |  |
| AMT | N/A | Active Management Technology (Intel® AMT 11.0) Support. |  |
| Audio Ports | 1 x 3.5 mm headset jack |  |  |
| Speakers | Two 2W internal speakers |  |  |
| Operating System Options | No Operating System Microsoft Windows 10 IoT Enterprise LTSC 64-bit |  |  |
| Languages | Windows 10 - 36 Language groups |  |  |
| Native (optimal) Resolution | 1920 x 1080 @ 60Hz |  |  |
| Control Buttons | Power Button |  |  |
| Aspect Ratio | 16:9 (standard) |  |  |
| Number of Colors | 15.6” models: 266,144 21.5” models: 16.7M |  |  |
| Brightness (typical) | 15.6” models: LCD panel: 300 nits; TouchPro PCAP: 270 nits 21.5” models: LCD panel: 250 nits; TouchPro PCAP: 225 nits |  |  |
| Response Time Total (typical) | 15.6” models: 25 ms 21.5” models: 14 ms |  |  |
| Viewing Angle (typical) | 15.6” models: 160° Horizontal / 160° Vertical 21.5” models: 178° Horizontal / 178° Vertical |  |  |
| Contrast Ratio (typical) | 15.6” models: 700:1 21.5” models: 1000:1 |  |  |
| Power Supply | External USB-C power supply brick AC input voltage: 100-240 VAC Input frequency: 50-60 Hz Maximum output power: 100W |  |  |
| Max Power Consumption (without peripherals) | 15.6” models: 30W 21.5” models: 39W | 15.6” models: 67W 21.5” models: 77W | 15.6” models: 73W 21.5” models: 83W |
| Energy Star Certified | Energy Star 7.1 |  |  |
| Shipping Box Dimensions (W x H x D) | 15.6” models: 18.19” x 8.5” x 13.57” / 462 mm x 216 mm x 346 mm 21.5” models: 24.61” x 7.72” x 17.2” / 625 mm x 196 mm x 437 mm |  |  |
| Weight | 15.6” models: 4.9 lbs / 2.2 kg 21.5” models: 11.5 lbs / 5.2 kg |  |  |
| Shipping Weight | 15.6” models: 9.5 lbs / 4.3 kg 21.5” models: 15.4 lbs / 7 kg |  |  |
| Temperature | Operating: 0°C to 35°C (32°F to 95°F) Storage: -20°C to 60°C (-4°F to 140°F) |  |  |
| Humidity (non-condensing) | Operating: 20% to 80% Storage: 5% to 95% (38.7°C max wet bulb temperature) |  |  |
| Regulatory approvals and declarations | UL/cUL , FCC, TUV, CE, CB, Mexico NOM, C-Tick/RCM, VCCI, Argentina S-Mark, CCC |  |  |
| Real-Time Clock | Replaceable CR2032 lithium metal battery for clock |  |  |
| Sealability | Touchscreen sealed to sub-bezel Touchscreen sealed to LCD |  |  |
| Power Cables | North American and European power cables included |  |  |
| Other Features | Serviceable internal components. Kensington lock. |  |  |
| Upgrade Options | 256GB 2.5” SATA SSD – Elo P/N: E061525 4GB DDR4 Memory SO-DIMM – Elo P/N: E275416 8GB DDR4 Memory SO-DIMM – Elo P/N: E275635 |  |  |
| Warranty | Three years (Extend two years with additional charge) 4 year warranty coverage - Elo P/N: E819598 5 year warranty coverage - Elo P/N: E819795 3 year warranty coverage + AUR - Elo P/N: E820186 4 year warranty coverage + AUR - Elo P/N: E820388 5 year warranty coverage + AUR - Elo P/N: E820582 |  |  |
| Optional Product Coverage | Advance Unit Replacement program (North America only) Extended Warranty (North America and EMEA only) |  |  |
| What’s in the box | - Windows I-Series Touchcomputer - North America Power Cord - EU Power Cord - Power Adapter – 100W - Cable cover locking screw - Quick Installation Guide (QIG) |  |  |
| Accessories |  |  |  |
| Stand, Mounts & Kits | - Shelf Mount Bracket – Elo P/N: E043382 - Floor Stand for 10/15/22” I-Series – Elo P/N: E048069 - Pole Mount – 6 inch – Elo P/N: E047458 - Pole Mount – 12 inch – Elo P/N: E047663 - Pole Mount – 18 inch – Elo P/N: E047864 - Pole Mount Bracket – Elo P/N: E045151 |  |  |
| Optional Peripherals | - Biometric Fingerprint Reader (FPR) – Elo P/N: E001001 - Magnetic Stripe Reader (MSR) – Elo P/N: E001002 - Customer Display (2x20 VFD) – Elo P/N: E001003 - 2D Barcode Reader (BCR) – Elo P/N: E926356 - EMV Cradle for eDynamo – Elo P/N: E375343 - EMV Cradle for Ingenico RP457 – Elo P/N: E710930 |  |  |

Part Number(s) for Windows I-Series 2.0 15.6-inch and 21.5-inch AiO Touchscreen Computer

| # Part Number | # Part Description | # Processor | # OS | # Memory | # Surface Treatment | # Touch Interface |
| --- | --- | --- | --- | --- | --- | --- |
| E691852 | ESY15i2-2UWB-0-MT-ZB-4G-1S-W1-64-BK-NS | J4105 | Window 10 | 4GB | Clear | USB |
| E692048 | ESY15i2-2UWB-0-MT-ZB-4G-1S-NO-64-BK-NS | J4105 | None | 4GB | Clear | USB |
| E850204 | ESY15i3-2UWB-0-MT-ZB-8G-1S-W1-64-BK-NS | i3-8100T | Window 10 | 8GB | Clear | USB |
| E850003 | ESY15i3-2UWB-0-MT-ZB-8G-1S-NO-64-BK-NS | i3-8100T | None | 8GB | Clear | USB |
| E692244 | ESY15i5-2UWB-0-MT-ZB-8G-1S-W1-64-BK-NS | i5-8500T | Window 10 | 8GB | Clear | USB |
| E692448 | ESY15i5-2UWB-0-MT-ZB-8G-1S-NO-64-BK-NS | i5-8500T | None | 8GB | Clear | USB |
| E692640 | ESY22i2-2UWB-0-MT-ZB-4G-1S-W1-64-BK-NS | J4105 | Window 10 | 4GB | Clear | USB |
| E692837 | ESY22i2-2UWB-0-MT-ZB-4G-1S-NO-64-BK-NS | J4105 | None | 4GB | Clear | USB |
| E850591 | ESY22i3-2UWB-0-MT-ZB-8G-1S-W1-64-BK-NS | i3-8100T | Window 10 | 8GB | Clear | USB |
| E850387 | ESY22i3-2UWB-0-MT-ZB-8G-1S-NO-64-BK-NS | i3-8100T | None | 8GB | Clear | USB |
| E693022 | ESY22i5-2UWB-0-MT-ZB-8G-1S-W1-64-BK-NS | i5-8500T | Window 10 | 8GB | Clear | USB |
| E693211 | ESY22i5-2UWB-0-MT-ZB-8G-1S-NO-64-BK-NS | i5-8500T | None | 8GB | Clear | USB |

Documentation List

| Document Number | Description |
| --- | --- |
| MS602478 | DIMENSIONAL DRAWING, WLIDS 2.0 |
| QIG600080 | QIG, WLIDS 2.0 |
| UM600348 | USER GUIDE, WLIDS 2.0 |
| UM600349 | USER GUIDE, WLIDS 2.0 EU SPANISH |
| UM600350 | USER GUIDE, WLIDS 2.0 LA SPANISH |
| UM600351 | USER GUIDE, WLIDS 2.0 FRENCH |
| UM600352 | USER GUIDE, WLIDS 2.0 ITALIAN |
| UM600353 | USER GUIDE, WLIDS 2.0 GERMAN |
| UM600354 | USER GUIDE, WLIDS 2.0 RUSSIAN |
| UM600355 | USER GUIDE, WLIDS 2.0 JAPANESE |
| UM600356 | USER GUIDE, WLIDS 2.0 SIM CHINESE |
| UM600357 | USER GUIDE, WLIDS 2.0 TRAD CHINESE |
| UM600358 | USER GUIDE, WLIDS 2.0 KOREAN |
| MD600103 | DECLARATIONS OF CONFORMITY, WLIDS 2.0 |

Elo Touch Solutions, Inc. its Affiliates (collectively “Elo”) reserves the right to change or update, without notice, any information contained herein; to change, without notice, the design, construction, materials, processing or specifications of any products; and to discontinue or limit production or distribution of any products.

Elo (logo), Elo Touch Solutions, Elo TouchSystems are trademarks